(c) Magnolia Microsystems 1982		Revised 12/9/82

Instructions for creating a new SASI module

-------------------------------------------

The required structure of a SASI module is as follows:

DRIVER MODULE (D320xxxx.HEX - direct result of assembling D320xxxx.ASM)

Controller text string

Drive text string for l.u.n. 0

Initialization data block for l.u.n. 0 (IDATxxxx.HEX)

Following this may be drive text strings and initialization data blocks for

up to three more logical units. In most cases only one logical unit will be

used. All the modules on this disk are set up for one logical unit. Thus, the

controller and drive text strings for a module are contained in one file called

M320xxxx.TXT.

Once the individual parts have been created, they may concatenated using PIP to

create a single file with the above structure and the name M320xxxx.HEX:

PIP M320xxxx.HEX=D320xxxx.HEX,M320xxxx.TXT,IDATxxxx.HEX

Step 1: Create new text strings

-------------------------------

The format for a controller text string is:

;"any text string describing the controller"

The format for a drive text string is:

;n"any text string describing logical unit n"

where n is a logical unit number from 0 to 3.

Note the use of the semi-colon and the double quote marks in the text

strings. These are required characters.

Step 2: Create new Initialization Data Blocks

---------------------------------------------

Each logical unit referenced by the new module (except for floppies) must have

its own initialization data block. This block of data gets written to sector 0

of the disk during a run of INITSASI and is accessed by the operating system

on the first select of a partition on that disk after cold boot (after warm

boot for removable hard disks). The data block contains hardware dependent

code, partitioning information and a boot loader routine (used only when

booting from the disk).

To create a new initialization data block, you may want to alter an already

existing IDATxxxx.ASM file, or start from scratch. Here is a sample

initialization data block (IDATS205.ASM) :

ORG	2280H

JMP	2480H

DB	0			; DRIVE/CONTROLLER CODE

DB	1			; CONTROL BYTE

DB	0,153,4,0,128,0,64,11	; DRIVE CHARACTERISTIC DATA

DB	0,0,0,0,0,0		; ASSIGN DRIVE TYPE COMMAND

DB	2			; NUMBER OF PARTITIONS

DB	0,0,0CH 		; PARTITION ADDRESS TABLE

DB	0,51H,4CH

DB	0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0

; DISK PARAMETER BLOCKS

DW	64			; SPT

DB	5,31,1			; BSH,BSM,EXM

DW	646-1,512-1		; DSM-1,DRM-1

DB	11110000B,0		; ALV0,ALV1

DW	0,2			; CKS,OFF

DB	00000010B,10000000B,0	; MODE BYTES

DB	0FFH,0FFH,0FFH		; MODE MASKS

DW	64

DB	5,31,1

DW	646-1,512-1

DB	11110000B,0

DW	0,2

DB	00000010B,10000000B,0

DB	0FFH,0FFH,0FFH

REPT	20+147

DB	0

ENDM

********** BOOT MODULE LOADER ROUTINE **********

The first two lines (ORG 2280H and JMP 2480H) must never be changed.

DRIVE/CONTROLLER CODE:

The format of the Drive/Controller code byte is as follows:

bits 5-7 : Controller code

bits 0-4 : Drive code

The following Drive/Controller combinations are supported by INITSASI:

Controller   Drive    Drive/Controller code

---------    -----    ---------------------

XEBEC	     ST-506   00000000

XEBEC	     CM5619   00000001

XEBEC	     CM5412   00000010

XEBEC	     MS-10    00000011

XEBEC	     other    00011111

DP-900	     DP-100   00100000

MRX 101D     MRX 101  01000000

other	     other    11111111

If your particular combination is not supported by INITSASI, you can still

use INITSASI to configure the drive. The only penalties will be that INITSASI

will not display expected format and disk test times, and will choose a

default interleave factor when formatting the drive. If your subsystem is not

listed above, you should use a value for Drive/Controller code different from

those listed (such as 11111111).

CONTROL BYTE:

The control byte for your controller can be found in your controller manual. It

may vary from drive to drive. In most cases it will be zero.

DRIVE CHARACTERISTIC DATA:

The eight bytes of Drive Characteristic Data are only used by the XEBEC

controller, and may be found in appendix A of the XEBEC S1410 Owner's Manual.

The first three bytes, though, are used by INITSASI to determine the number of

tracks on the drive for track verification, and should be set correctly no

matter what controller is being used. The first two bytes are the number of

cylinders on the drive (high order byte first) and the third byte is the

number of heads.

ASSIGN DRIVE TYPE COMMAND:

The six bytes of Assign Drive Type Command are used only by the DP-900

controller, and may be found in the Controller Specification Manual for the

DP-900 controller. If your controller is other than the DP-900, these bytes

must all be zero.

NUMBER OF PARTITIONS:

The next byte is the number of partitions on this logical unit. Any value from

0 to 8 is allowed.

PARTITION ADDRESS TABLE:

The next 27 bytes are the partition address table, with three bytes for each

partition indicating the starting sector on the drive for the partition. The

partition addresses for unused partitions may be set to zeroes. The partition

address for partition 0 must 0,0,0CH. The rest of the addresses must be

calculated after the disk parameter blocks have been set up.

DISK PARAMETER BLOCKS:

The next 189 bytes are the disk parameter blocks, consisting of 21 bytes for

each partition. See the Digital Research CP/M 2.2 Alteration Guide for

information on setting up disk parameter blocks. The blocks can be set up

however you want (providing, of course, that you do not allocate more space

than your drive contains) with the exception that the first entry, sectors

per track, must be 64. This has nothing to do with the number of physical

sectors per track on your drive.

MODE BYTES AND MODE MASKS:

The last six bytes of each disk parameter block are the mode bytes and mode

masks. The important bits in the mode bytes are bits 0 and 1 of the first mode

byte, which indicate the actual physical sector size of your drive, and bit 7

of the second mode byte, which tells the MODE program to ignore the mode bytes.

The mode mask bits must all be set, preventing any program from altering the

mode bytes.

CALCULATING PARTITION ADDRESSES:

Using the information in the disk parameter block for a partition, the

partition address can be calculated. Using the Alteration Guide again,

determine the block size, and then the number of sectors per block (SPB). SPB

can be determined knowing that there are eight sectors in 1K. Now calculate the

total sectors on the partition using this equation:

Total Sectors = SPT * OFF + SPB * DSM

Having calculated the total sectors on partition 0, add this to the starting

address of partition 0 to get the starting address of partition 1, and so on

until you have a starting address for each partition.

BOOT LOADER ROUTINE:

The last part of the initialization data block is the boot loader routine.

If you are creating a new data block, the boot loader source code, L320.ASM

must be included here.

ASSEMBLE THE FILE:

The final step of creating the initialization data block is to assemble the

code using MAC.

Step 3: Create a new driver module:

----------------------------------

The driver module has no controller specific code in it. The parameters that

must change are related to number of partitions, partition size, and whether

the disk is fixed or removable. In version 2.243 and later, there is nothing

in the driver module specific to partition size. These modules are named with

two characters only: F or R for fixed or removable, and the number of

partitions.

The best way to create a new driver module is to edit an already existing one.

Find the section, near the beginning if the D320xxx.ASM file, that appears as

follows:

***************************************************

** OVERLAY MODULE INFORMATION ON BIOS

***************************************************

ORG	PATCH

DS	51		; BIOS JUMP TABLE

DISK$STAT	DB	0

DS	8

DRIV0	EQU	50

MIXER:	DB	50,51		; PHYSICAL DRIVES 50,51

DS	14		; LOGICAL-PHYSICAL DRIVE TABLE

DRIVE$BASE:

DB	50,52		; DRIVE MODULE BASE TABLE

DW	MBASE

DS	28

CBIOS:	DS	3

NEWBAS	DS	2

NEWDSK	DS	1

NEWTRK	DS	1

NEWSEC	DS	1

HRDTRK	DS	2

DMAA	DS	2

***************************************************

The values to be altered here are MIXER and DRIVE$BASE.

MIXER:

Up to nine numbers starting with 50 may be entered here, one for each partition

or floppy disk to be accessed by this module. The following DS statement must

be set so that 16 bytes all together are defined for MIXER.

DRIVE$BASE:

The first byte must be 50. The second byte must be the highest drive number you

assigned in MIXER plus 1.

The next section of code in the driver module appears as follows:

***************************************************

** START OF RELOCATABLE DISK I/O MODULE

***************************************************

ORG	MBASE		; START OF MODULE

JMP	SEL$SASI

JMP	READ$SASI

JMP	WRITE$SASI

;	TEXT

DB	'77320 SASI Interface (2 partitions) v2.24'

DW	VERS,'$'

DPH0:	DW	0,0,0,0,DIRBUF,DPB0,CSV0,ALV0

DPH1:	DW	0,0,0,0,DIRBUF,DPB1,CSV1,ALV1

DPH2:	DW	0,0,0,0,DIRBUF,DPB2,CSV2,ALV2

DPH3:	DW	0,0,0,0,DIRBUF,DPB3,CSV3,ALV3

DPH4:	DW	0,0,0,0,DIRBUF,DPB4,CSV4,ALV4

DPH5:	DW	0,0,0,0,DIRBUF,DPB5,CSV5,ALV5

DPH6:	DW	0,0,0,0,DIRBUF,DPB6,CSV6,ALV6

DPH7:	DW	0,0,0,0,DIRBUF,DPB7,CSV7,ALV7

DPH8:	DW	0,0,0,0,DIRBUF,DPB8,CSV8,ALV8

CNUM:	DB	0		; CONTROLLER NUMBER 0

;	SECTOR DEFINITION/TRANSLATION TABLE

;		--ADDRESS--,   FLAG BYTE

DDEFTBL:DB	0,   0,   0,   0

DB	0,   0,   0,   1

DB	0,   0,   0,   0

DB	0,   0,   0,   0

DB	0,   0,   0,   0

DB	0,   0,   0,   0

DB	0,   0,   0,   0

DB	0,   0,   0,   0

DB	0,   0,   0,   0

The TEXT string may be changed as desired, providing that the number of bytes

in the string is not altered. Otherwise, the relocation bit map at the end of

the module must be changed

DDEFTBL:

There are four bytes here for each partition. The first three bytes are for the

partition address, to be extracted from the initialization block on the first

select of the partition. The first three bits of the first byte, though, must

indicate the logical unit on which the partition is located. This will be zero

in most cases. The fourth byte for each partition is a flag byte, defined as

follows:

BIT 7 = INITIALIZATION FLAG -- always 0

BIT 6 = FLOPPY DISK FLAG -- 1 for floppy, 0 for hard

BIT 5 = REMOVABLE MEDIA FLAG -- 1 for removable, 0 for fixed

BIT 4 (SPARE)

BITS 0-3 = PARTITION NUMBER -- relative to logical unit

Now go to the end of the driver module, to find code that looks like this:

********************************************************

** BUFFERS

********************************************************

ORG	BUFFER

HSTBUF: DS	512

CSV0:	DS	0

ALV0:	DS	81

CSV1:	DS	0

ALV1:	DS	81

CSV2:	DS	0

ALV2:	DS	0

CSV3:	DS	0

ALV3:	DS	0

CSV4:	DS	0

ALV4:	DS	0

CSV5:	DS	0

ALV5:	DS	0

CSV6:	DS	0

ALV6:	DS	0

CSV7:	DS	0

ALV7:	DS	0

CSV8:	DS	0

ALV8:	DS	0

**********************************************************

BUFLEN	EQU	$-BUFFER

END

BUFFER SIZES:

Define one ALV buffer and one CSV buffer for each partition. If the media is

removable, the space defined for the CSV buffer for any partition must be the

same (or larger) than the value defined for CKS when you were setting up disk

parameter blocks. If the media is fixed, the CSV buffers should have a length

of 0. The length of the ALV buffer for a partition is determined by dividing

the DSM value for that partition (again from the disk parameter block) by 8. In

version 2.243 or later, all ALV buffers are of length 256, and CSV buffers for

removable media are of length 128.

ASSEMBLE THE FILE:

The last step in creating a new driver module is to assemble the file using

MAC.

Step 4: Put all the pieces together:

-----------------------------------

All the files just created may concatenated using PIP to create one file of

the structure described at the beginning of this document. Now this file can

be used to initialize the drive using INITSASI. Either this file or the driver

module, D320xxxx.HEX, can be used to link the SASI subsystem into a MOVCPM

file. Note that LINK ignores the text strings and the initialization data

block.