(c) Magnolia Microsystems 1982

INITSASI Error Messages

-----------------------

Errors on reading in the M320xxxx.HEX file:

------------------------------------------

These error messages will occur directly following the sign-on message, and

will be followed by a warm boot. Information from the HEX file and the menu

of options will not be disaplyed if any of these errors occur.

No file name specified

will occur if no file name was typed in following 'INITSASI'.

d:filename.HEX not found

will occur if there is no file of the type HEX and the file name and drive

specified by the user.

d:filename.hex is not a recognizable SASI-bus disk I/O module

can occur for several different reasons:

1)End of file is reached before all the required parts of the file have been

found.

2)The controller number specified in the driver module is not in the range

0-7.

3)The first text string found in the file does not begin with a double quote

4)The second text string found does not begin with a logical unit number in

the range 0-3, and then a double quote

5)An initialization data block does not exist for logical unit 0

6)The initialization data block for logical unit 0 does not begin at address

2280H with the code C3 80 24

7)The initialization data block for logical unit 0 is too long or too short

Errors that can occur on any menu option:

----------------------------------------

Unless option 4, Exit to CP/M, is chosen, certain tests will be performed

before the particular function chosen is carried out. These tests can result

in the following error messages.

SW501 SETTING INCORRECT - NO PORT SELECTED

can occur if Switch 501 does not indicate that either port 78H or port 7CH is

set up for the SASI bus.

CONTROLLER NOT READY

can occur if for some reason the controller does not respond when it is

requested through either port 78H or port 7CH. Possibly the power is not turned

on, or the controller is not attached to the interface card correctly.

SW501 SETTING DOES NOT MATCH PORT CHOICE

can occur if the controller does not respond when requested through the port

selected by Switch 501, but does respond when requested through the other port.

Error on logical unit select:

----------------------------

Menu options 2 and 3 access drives, so when these options are selected,

INITSASI will ask the user to select a logical unit. If the logical unit is

changed from what it was before, a new initialization data block must be read

from the HEX file.

Initialization data for this drive is BAD

can occur for the following reasons:

1)The initialization data block does not exist for the requested logical unit

2)The initialization data block for the logical unit does not begin at address

2280H with the code C3 80 24

3)The initialization data block for the logical unit is too long or too short

Errors on selection of a diagnostic option:

------------------------------------------

Before any diagnostic command is executed, INITSASI checks the Drive/Controller

code byte in the initialization data block to see if the controller is a XEBEC

S1410. If it is another controller, the following message will be displayed,

and the command will not be executed:

Controller does not support software diagnostics

If the controller is a XEBEC, the test will be performed, and the following

error messages can occur:

CONTROLLER NOT READY

NO INDEX SIGNAL

SEEK NOT COMPLETED

DRIVE FAULT

DRIVE NOT READY

TRACK 0 NOT FOUND (Check Configuration Jumpers)

ID READ ERROR

UNCORRECTABLE DATA ERROR

NO ID ADDRESS MARK

SECTOR NOT FOUND

SEEK ERROR

CORRECTABLE DATA ERROR

BAD TRACK

FORMAT ERROR

INVALID COMMAND

ILLEGAL DISK ADDRESS

RAM ERROR

PROGRAM MEMORY CHECKSUM ERROR

ECC POLYNOMIAL ERROR

UNDEFINED ERROR

Messages appearing after INSPECTING DRIVE ...:

---------------------------------------------

Drive not ready

is self explanatory.

An attempt is made to read sector 0 of the disk.  If the read is sucessful,

the data is checked to see if it looks like an initialization data block.

If the sector is readable and the first three bytes are C3 80 24, the following

message will be displayed:

Drive has been previously configured and may contain data.

Continuing will DESTROY any data on the drive.

Do you want to DESTROY the data ? (Y or N) N

If the user types 'N' (or <CR>), this message will be replaced by:

Format not attempted

and the function will be discontinued. If the user types 'Y', the message will

be replaced by:

Any existing data Destroyed

and then INITSASI will attempt to format the drive.

Errors on formatting of drive:

-----------------------------

After the drive is formatted, the tracks will be checked one by one (unless the

controller is a Z67, which does not support track checking). If verify errors

occur, the following display will be shown:

First bad track: xxxx

Last bad track:  yyyy

Number of bad tracks: zzzz

Error on writing of data:

------------------------

During the writing of the initialization data block or the clearing of the

directories, the following error may occur:

Write error